PROCESS DESIGN BASIS
50 MMSCFD Amine-Based AGRU

1. PROJECT DESCRIPTION

1.1 Objective
Design an Amine-Based Acid Gas Removal Unit (AGRU) to remove H₂S and CO₂ from sour natural gas to meet pipeline quality specifications.

The unit shall:

- Process 50 MMSCFD sour gas

- Reduce H₂S to < 4 ppmv

- Reduce CO₂ to < 2 mol%

- Operate continuously under steady-state condition

## 1.2 Process Overview

The AGRU shall use aqueous Diethanolamine (DEA) solution as the solvent.

Process sections:

- Absorption Section (High Pressure)

- Flash Section

- Regeneration Section (Stripper)

- Amine Cooling & Circulation Section

Operation philosophy: Continuos cyclic solvent regeneration system.

# Design Basis

## 2.1 Feed Gas Conditions

Parameter                                                                                                 Value

Flowrate                                                                                                 50 MMSCFD

Pressure                                                                                                  45 bar

Temperature                                                                                            40°C

Phase                                                                                                      Vapor

## 2.2 Feed Composition (Mole %)

Component                                                                                              Mole (%)

CH4                                                                                                                                                                     89.5

CO2                                                                                                                                                                    6.0

H2S                                                                                                               2.5

N2                                                                                                                                                                           2.0

## 2.3 Product Gas Specification

Parameter                                                                                                  Requirement

H2S                                                                                                                 < 4 ppmv

CO2                                                                                                                                                                         < 2 mol %

N2                                                                                                                                                                                 Saturated

## 3. SOLVENT BASIS

Parameter                                                                                                       Value

Solvent Type                                                                                                DEA

Concentration                                                                                               30 wt%

Lean Loading Target                                                                 0.05 mol acid gas/mol amine

Reach Loading Target                                                               0.40 mol/mol

# 4. DESIGN PHILOSOPHY

## 4.1 Absorber

- Counter-current tray column

- High pressure operation (~45 bar)

- Designed at 80% flooding

## 4.2 Regenerator

- Low pressure (~1.5–2 bar)

- Steam reboiler

- Overhead condenser with reflux

## 5. Operating Philosophy

- Continuous operation (24/7)

- Automatic pressure control in absorber

- Level control in flash drum and stripper bottom

- Reboiler temperature controlled via steam flow

## 6. DESIGN CODES & STANDARD

- ASME Section VIII (Pressure Vessels)

- API 520/521 (Relief)

- ANSI B16.5 (Flanges)

- ASTM material standards

## 7.UTILITIES BASIS

Utility                                                                                                                               Assumption

Steam                                                                                                                     3 barg saturated

Cooling Water                                                                                                       30°C Inlet

Electricity                                                                                                              415 V

Instrument Air                                                                                                        7 barg

## 8. SAFETY BASIS

- H₂S classified as toxic gas

- Unit area classified hazardous

- Relief system required for:

Blocked outlet

Fire Case

Reboiler tube rupture

- Closed drain system required

## 9. DESIGN MARGINS

- Flow Rate : +10%

- Equipment Duty : +10%

- Column Diameter: 20% above minimum